============================================================

A题 电机控制实验装置 —— 全部源代码

Motor: PA0(PWM)->ENA  PB12->IN1  PB13->IN2

============================================================

============================================================

User/main.c

============================================================

#include "stm32f10x.h"

#include "Delay.h"

#include "OLED.h"

#include "Timer.h"

#include "Key.h"

#include "Motor.h"

#include "Encoder.h"

#include "Serial.h"

float Target_RPM, Actual_RPM, PID_Out;

float Kp, Ki, Kd;

uint8_t MotorRun;

float Error0, Error1, ErrorInt;

volatile uint8_t TxFlag;

int main(void)

{

OLED_Init();

Key_Init();

Motor_Init();

Encoder_Init();

Serial_Init();

Timer_Init();

Kp = 0.1;

Ki = 0.005;

Kd = 0.4;

Target_RPM = 1500;

MotorRun = 0;

OLED_Printf(0, 0, OLED_8X16, "A-Ti Motor Ctrl");

OLED_Update();

Serial_Printf("Target,Actual,Output\r\n");

while (1)

{

uint8_t kn = Key_GetNum();

if (kn == 1)

{

MotorRun = !MotorRun;

if (MotorRun) ErrorInt = 0;

else Motor_SetPWM(0);

}

if (kn == 2) { Target_RPM += 10; if (Target_RPM > 3000) Target_RPM = 3000; }

if (kn == 3) { Target_RPM -= 10; if (Target_RPM < 0) Target_RPM = 0; }

/* ── 串口接收 ── */

if (Serial_GetRxFlag())

{

ParseCmd(Serial_RxPacket);

}

/* ── 串口发送 ── */

if (TxFlag)

{

TxFlag = 0;

Serial_Printf("%.1f,%.1f,%.1f\r\n", Target_RPM, Actual_RPM, PID_Out);

}

OLED_Printf(0, 16, OLED_8X16, "Tar:%5.0f RPM", Target_RPM);

OLED_Printf(0, 32, OLED_8X16, "Act:%5.0f RPM", Actual_RPM);

OLED_Printf(0, 48, OLED_8X16, "%s %s",

MotorRun ? "RUN " : "STOP", "");

OLED_Update();

}

}

void TIM1_UP_IRQHandler(void)

{

static uint16_t Count;

if (TIM_GetITStatus(TIM1, TIM_IT_Update) == SET)

{

Key_Tick();

Encoder_Tick();

Count++;

if (Count >= 20)

{

Count = 0;

int16_t Speed = Encoder_Get();

if (Speed >= 0) Actual_RPM = Speed;

if (MotorRun)

{

Error1 = Error0;

Error0 = Target_RPM - Actual_RPM;

if (Ki != 0)

{

ErrorInt += Error0;

if (ErrorInt > 90/Ki) ErrorInt = 90/Ki;

if (ErrorInt < -90/Ki) ErrorInt = -90/Ki;

}

PID_Out = Kp * Error0 + Ki * ErrorInt + Kd * (Error0 - Error1);

if (PID_Out > 100)  PID_Out = 100;

if (PID_Out < -100) PID_Out = -100;

Motor_SetPWM((int8_t)PID_Out);

}

else

{

Motor_SetPWM(0);

}

}

{

static uint16_t TxCount;//串口发送

TxCount++;

if (TxCount >= 100)

{

TxCount = 0;

TxFlag = 1;

}

}

TIM_ClearITPendingBit(TIM1, TIM_IT_Update);

}

}

============================================================

Hardware/Motor.c

============================================================

#include "stm32f10x.h"                  // Device header

#include "PWM.h"

/*

* PA0 (TIM2_CH1 PWM) → ENA    速度控制

* PB12 (GPIO)         → IN1    方向控制

* PB13 (GPIO)         → IN2    方向控制

* ENA 不戴跳线帽

*/

void Motor_Init(void)

{

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOB, ENABLE);

GPIO_InitTypeDef GPIO_InitStructure;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_PP;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_12 | GPIO_Pin_13;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOB, &GPIO_InitStructure);

PWM_Init();

}

void Motor_SetPWM(int8_t Speed)

{

if (Speed >= 0)

{

GPIO_SetBits(GPIOB, GPIO_Pin_12);		// IN1 = 高

GPIO_ResetBits(GPIOB, GPIO_Pin_13);		// IN2 = 低

}

else

{

GPIO_ResetBits(GPIOB, GPIO_Pin_12);		// IN1 = 低

GPIO_SetBits(GPIOB, GPIO_Pin_13);		// IN2 = 高

Speed = -Speed;

}

if (Speed > 100) Speed = 100;

PWM_SetCompare1(Speed);					// ENA PWM

}

============================================================

Hardware/Motor.h

============================================================

#ifndef __MOTOR_H

#define __MOTOR_H

void Motor_Init(void);

void Motor_SetPWM(int8_t PWM);

#endif

============================================================

Hardware/Encoder.c

============================================================

#include "stm32f10x.h"                  // Device header

/*

* 测速传感器：槽型光电开关，电机每转 1 个脉冲

* 输出经 8050 NPN 三极管开漏 → 10K 上拉 → J2 Pin1

* 接 STM32 PA6 (TIM3_CH1)，输入捕获测周期

*

* TIM3: 72MHz / 720 = 100kHz (10us tick)

* 2 秒无脉冲判为 0 RPM，约 30 RPM 以下视为 0

*/

static volatile uint16_t Period;

static volatile uint8_t  Updated;

static volatile uint16_t Timeout;	// 无脉冲计数器，TIM1 1ms 递增

void Encoder_Init(void)

{

RCC_APB1PeriphClockCmd(RCC_APB1Periph_TIM3, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA, ENABLE);

GPIO_InitTypeDef GPIO_InitStructure;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IPU;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_6;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

TIM_TimeBaseInitTypeDef TIM_TimeBaseInitStructure;

TIM_TimeBaseInitStructure.TIM_ClockDivision = TIM_CKD_DIV1;

TIM_TimeBaseInitStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInitStructure.TIM_Period = 65536 - 1;

TIM_TimeBaseInitStructure.TIM_Prescaler = 720 - 1;		// 100kHz

TIM_TimeBaseInitStructure.TIM_RepetitionCounter = 0;

TIM_TimeBaseInit(TIM3, &TIM_TimeBaseInitStructure);

TIM_ICInitTypeDef TIM_ICInitStructure;

TIM_ICInitStructure.TIM_Channel = TIM_Channel_1;

TIM_ICInitStructure.TIM_ICFilter = 0x0F;

TIM_ICInitStructure.TIM_ICPolarity = TIM_ICPolarity_Rising;

TIM_ICInitStructure.TIM_ICPrescaler = TIM_ICPSC_DIV1;

TIM_ICInitStructure.TIM_ICSelection = TIM_ICSelection_DirectTI;

TIM_ICInit(TIM3, &TIM_ICInitStructure);

TIM_ITConfig(TIM3, TIM_IT_CC1, ENABLE);

NVIC_InitTypeDef NVIC_InitStructure;

NVIC_InitStructure.NVIC_IRQChannel = TIM3_IRQn;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 1;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 0;

NVIC_Init(&NVIC_InitStructure);

TIM_Cmd(TIM3, ENABLE);

}

int16_t Encoder_Get(void)

{

int16_t Speed;

uint16_t p;

if (Timeout > 2000)			// 2 秒无脉冲 → 0 RPM

{

return 0;

}

if (Updated == 0)

{

return -1;//代表无有效转速

}

Updated = 0;

p = Period;

if (p == 0) return 0;

Speed = 60 * 100000 / p;		// RPM = 60 / (period_seconds)

return Speed;

}

void Encoder_Tick(void)				// 由 TIM1 每 1ms 调用

{

Timeout++;

}

void TIM3_IRQHandler(void)

{

static uint16_t LastCap;

uint16_t CurCap;

if (TIM_GetITStatus(TIM3, TIM_IT_CC1) == SET)

{

CurCap = TIM_GetCapture1(TIM3);//读取本次捕获定时器的CNT值

Period = CurCap - LastCap;//算两次脉冲的间隔

LastCap = CurCap;//更新上次捕获值为当前

Updated = 1;//可计算转速

Timeout = 0;				// 收到脉冲，清超时

TIM_ClearITPendingBit(TIM3, TIM_IT_CC1);

}

}

============================================================

Hardware/Encoder.h

============================================================

#ifndef __ENCODER_H

#define __ENCODER_H

void Encoder_Init(void);

int16_t Encoder_Get(void);

void Encoder_Tick(void);

#endif

============================================================

Hardware/PWM.c

============================================================

#include "stm32f10x.h"                  // Device header

void PWM_Init(void)

{

RCC_APB1PeriphClockCmd(RCC_APB1Periph_TIM2, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA, ENABLE);

GPIO_InitTypeDef GPIO_InitStructure;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF_PP;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_0;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

TIM_InternalClockConfig(TIM2);

TIM_TimeBaseInitTypeDef TIM_TimeBaseInitStructure;

TIM_TimeBaseInitStructure.TIM_ClockDivision = TIM_CKD_DIV1;

TIM_TimeBaseInitStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInitStructure.TIM_Period = 100 - 1;

TIM_TimeBaseInitStructure.TIM_Prescaler = 36 - 1;

TIM_TimeBaseInitStructure.TIM_RepetitionCounter = 0;

TIM_TimeBaseInit(TIM2, &TIM_TimeBaseInitStructure);

TIM_OCInitTypeDef TIM_OCInitStructure;

TIM_OCStructInit(&TIM_OCInitStructure);

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_High;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_Pulse = 0;

TIM_OC1Init(TIM2, &TIM_OCInitStructure);

TIM_Cmd(TIM2, ENABLE);

}

void PWM_SetCompare1(uint16_t Compare)

{

TIM_SetCompare1(TIM2, Compare);

}

============================================================

Hardware/PWM.h

============================================================

#ifndef __PWM_H

#define __PWM_H

void PWM_Init(void);

void PWM_SetCompare1(uint16_t Compare);

#endif

============================================================

Hardware/Key.c

============================================================

#include "stm32f10x.h"                  // Device header

#include "Delay.h"

uint8_t Key_Num;

void Key_Init(void)

{

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOB, ENABLE);

GPIO_InitTypeDef GPIO_InitStructure;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IPU;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_10 | GPIO_Pin_11;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOB, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_11;

GPIO_Init(GPIOA, &GPIO_InitStructure);

}

uint8_t Key_GetNum(void)

{

uint8_t Temp;

if (Key_Num)

{

Temp = Key_Num;

Key_Num = 0;

return Temp;

}

return 0;

}

uint8_t Key_GetState(void)

{

if (GPIO_ReadInputDataBit(GPIOB, GPIO_Pin_10) == 0) return 1;

if (GPIO_ReadInputDataBit(GPIOB, GPIO_Pin_11) == 0) return 2;

if (GPIO_ReadInputDataBit(GPIOA, GPIO_Pin_11) == 0) return 3;

return 0;

}

void Key_Tick(void)

{

static uint8_t Count;

static uint8_t CurrState, PrevState;

Count ++;

if (Count >= 50)

{

Count = 0;

PrevState = CurrState;

CurrState = Key_GetState();

if (CurrState != 0 && PrevState == 0)

{

Key_Num = CurrState;

}

}

}

============================================================

Hardware/Key.h

============================================================

#ifndef __KEY_H

#define __KEY_H

void Key_Init(void);

uint8_t Key_GetNum(void);

void Key_Tick(void);

#endif

============================================================

Hardware/Serial.c

============================================================

#include "stm32f10x.h"

#include <stdio.h>

#include <stdarg.h>

#include <string.h>

#include <stdlib.h>

#include "Motor.h"

char Serial_RxPacket[32];

uint8_t Serial_RxFlag;

extern float Target_RPM, Kp, Ki, Kd;

extern uint8_t MotorRun;

extern float ErrorInt;

void Serial_Printf(char *format, ...);

void ParseCmd(char *s)

{

if (strcmp(s, "START") == 0)//完整匹配的指令

{

MotorRun = 1; ErrorInt = 0;

Serial_Printf("OK START\r\n");

}

else if (strcmp(s, "STOP") == 0)

{

MotorRun = 0; Motor_SetPWM(0);

Serial_Printf("OK STOP\r\n");

}

else if (strcmp(s, "UP") == 0)

{

Target_RPM += 10;

if (Target_RPM > 3000) Target_RPM = 3000;

Serial_Printf("OK TARGET=%.0f\r\n", Target_RPM);

}

else if (strcmp(s, "DOWN") == 0)

{

Target_RPM -= 10;

if (Target_RPM < 0) Target_RPM = 0;

Serial_Printf("OK TARGET=%.0f\r\n", Target_RPM);

}

else if (strncmp(s, "SPD=", 4) == 0)//只对比前四个字符

{

Target_RPM = atof(s + 4);//atof：把字符串数字转成浮点

if (Target_RPM < 0) Target_RPM = 0;

if (Target_RPM > 3000) Target_RPM = 3000;

Serial_Printf("OK TARGET=%.0f\r\n", Target_RPM);

}

else if (strncmp(s, "KP=", 3) == 0)

{

Kp = atof(s + 3);

Serial_Printf("OK KP=%.3f\r\n", Kp);

}

else if (strncmp(s, "KI=", 3) == 0)

{

Ki = atof(s + 3); ErrorInt = 0;

Serial_Printf("OK KI=%.3f\r\n", Ki);

}

else if (strncmp(s, "KD=", 3) == 0)

{

Kd = atof(s + 3);

Serial_Printf("OK KD=%.3f\r\n", Kd);

}

}

void Serial_Init(void)

{

RCC_APB2PeriphClockCmd(RCC_APB2Periph_USART1, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA, ENABLE);

GPIO_InitTypeDef GPIO_InitStructure;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF_PP;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_9;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IPU;

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_10;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

USART_InitTypeDef USART_InitStructure;

USART_InitStructure.USART_BaudRate = 115200;

USART_InitStructure.USART_HardwareFlowControl = USART_HardwareFlowControl_None;

USART_InitStructure.USART_Mode = USART_Mode_Tx | USART_Mode_Rx;

USART_InitStructure.USART_Parity = USART_Parity_No;

USART_InitStructure.USART_StopBits = USART_StopBits_1;

USART_InitStructure.USART_WordLength = USART_WordLength_8b;

USART_Init(USART1, &USART_InitStructure);

USART_ITConfig(USART1, USART_IT_RXNE, ENABLE);

NVIC_PriorityGroupConfig(NVIC_PriorityGroup_2);

NVIC_InitTypeDef NVIC_InitStructure;

NVIC_InitStructure.NVIC_IRQChannel = USART1_IRQn;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 1;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 1;

NVIC_Init(&NVIC_InitStructure);

USART_Cmd(USART1, ENABLE);

}

static void Serial_SendByte(uint8_t Byte)

{

USART_SendData(USART1, Byte);

while (USART_GetFlagStatus(USART1, USART_FLAG_TXE) == RESET);

}

static void Serial_SendString(char *String)

{

uint8_t i;

for (i = 0; String[i] != '\0'; i ++)

{

Serial_SendByte(String[i]);

}

}

void Serial_Printf(char *format, ...)

{

char String[100];

va_list arg;

va_start(arg, format);

vsprintf(String, format, arg);

va_end(arg);

Serial_SendString(String);

}

uint8_t Serial_GetRxFlag(void)

{

if (Serial_RxFlag)

{

Serial_RxFlag = 0;

return 1;

}

return 0;

}

void USART1_IRQHandler(void)

{

static uint8_t RxState = 0;

static uint8_t pRxPacket = 0;

if (USART_GetITStatus(USART1, USART_IT_RXNE) == SET)

{

uint8_t RxData = USART_ReceiveData(USART1);

if (RxState == 0)

{

if (RxData == '@' && Serial_RxFlag == 0)

{

RxState = 1;

pRxPacket = 0;

}

}

else if (RxState == 1)

{

if (RxData == '#')

{

RxState = 0;

Serial_RxPacket[pRxPacket] = '\0';

Serial_RxFlag = 1;

}

else

{

Serial_RxPacket[pRxPacket] = RxData;

pRxPacket ++;

}

}

USART_ClearITPendingBit(USART1, USART_IT_RXNE);

}

}

============================================================

Hardware/Serial.h

============================================================

#ifndef __SERIAL_H

#define __SERIAL_H

#include <stdio.h>

extern char Serial_RxPacket[];

extern uint8_t Serial_RxFlag;

void Serial_Init(void);

void Serial_Printf(char *format, ...);

uint8_t Serial_GetRxFlag(void);

void ParseCmd(char *s);

#endif

============================================================

System/Timer.c

============================================================

#include "stm32f10x.h"                  // Device header

void Timer_Init(void)

{

RCC_APB2PeriphClockCmd(RCC_APB2Periph_TIM1, ENABLE);

TIM_InternalClockConfig(TIM1);

TIM_TimeBaseInitTypeDef TIM_TimeBaseInitStructure;

TIM_TimeBaseInitStructure.TIM_ClockDivision = TIM_CKD_DIV1;

TIM_TimeBaseInitStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInitStructure.TIM_Period = 1000 - 1;

TIM_TimeBaseInitStructure.TIM_Prescaler = 72 - 1;

TIM_TimeBaseInitStructure.TIM_RepetitionCounter = 0;

TIM_TimeBaseInit(TIM1, &TIM_TimeBaseInitStructure);

TIM_ClearFlag(TIM1, TIM_FLAG_Update);

TIM_ITConfig(TIM1, TIM_IT_Update, ENABLE);

NVIC_PriorityGroupConfig(NVIC_PriorityGroup_2);

NVIC_InitTypeDef NVIC_InitStructure;

NVIC_InitStructure.NVIC_IRQChannel = TIM1_UP_IRQn;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 2;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 1;

NVIC_Init(&NVIC_InitStructure);

TIM_Cmd(TIM1, ENABLE);

}

============================================================

System/Timer.h

============================================================

#ifndef __TIMER_H

#define __TIMER_H

void Timer_Init(void);

#endif

============================================================

System/Delay.c

============================================================

#include "stm32f10x.h"

void Delay_us(uint32_t xus)

{

SysTick->LOAD = 72 * xus;

SysTick->VAL = 0x00;

SysTick->CTRL = 0x00000005;

while(!(SysTick->CTRL & 0x00010000));

SysTick->CTRL = 0x00000004;

}

void Delay_ms(uint32_t xms)

{

while(xms--)

{

Delay_us(1000);

}

}

void Delay_s(uint32_t xs)

{

while(xs--)

{

Delay_ms(1000);

}

}

============================================================

System/Delay.h

============================================================

#ifndef __DELAY_H

#define __DELAY_H

void Delay_us(uint32_t us);

void Delay_ms(uint32_t ms);

void Delay_s(uint32_t s);

#endif